# Reconfiguring a Johnstown Motherboard After Battery Replacement

Ver 1.0  	3 June 2020

After replacing the CR2032 CMOS battery, you will need to properly configure the BIOS in the motherboard. This is done as follows:

Connect the 12V power supply to the rear of the 1100/1200.  A standby LED on the motherboard should illuminate.

Remove the ribbon header for the front panel indicators from the header on the motherboard (feature V in board drawing at below).

Momentarily touch a screwdriver to the two pins with RED background on this header. (This acts as a power-on switch.)

- Note:  If the unit fails to boot up, it may be necessary to reset the BIOS CMOS.  See procedure at the end of this document for steps in accomplishing this.

Press function key F2 immediately upon seeing the Intel logo screen.

In the BIOS setup, press the right arrow () key once to select the Advanced menu.

Press the down arrow key to select the Drive Configuration option.

The first item in the menu, ATA/IDE Mode, should be selected.  If not use the up/down arrow keys to select.  The value should be [Native].  If not, press the (+) key on the keypad until the option “[Native]’ is displayed.

Press the down arrow key until the option Configure SATA is selected.

Press the plus (+) key  on the keypad until the value is “[AHCI]”.

Press the Esc key.

Press the right arrow key two times to select the Power menu.

The first item in the menu, After Power Failure, should be selected by default.  If not, use the up/down arrow keys to select.

If not already set, press the plus (+) key on the keypad until the option “[Power on]” is displayed.

Press the F10 key to save the settings.

Press the [Enter] key on the keyboard and select Yes in the Yes/No dialog that appears.