AtlasIED AS Series BIM file Usage Notes

There are two BIM files for the FS series loudspeakers;

- Atlas FS Series with Yoke Bracket Assembly_rvt2022_rev0

- Atlas FS Series with Pivoting Wall Bracket Assembly_rvt2022_rev0

These are “assembly” families which include all models of the FS series loudspeakers in white and black and the associated mounting brackets. They are also “Face Based” and can be inserted on any Face in your project, including faces of any linked models. The zip file includes the Family and a text file with the same name. This text file must be saved in the same location as the BIM file in order for the library function to work correctly.

When you first load the family into your project, you will see a dialog box listing all the available models (Types) in the family along with all the data for each type.

The user may select any combination of Types desired for the project. This may be just one or all, depending on your needs. This helps to reduce the file size of your project model. Once loaded, you have the option to insert the Family on a “Vertical Face, “Face” or “Workplane”.

Note that the Families will show up under “Communication Devices” in your project “Families” Browser. You will also find nested Pivoting Wall Bracket Families under “Generic Models”. Please avoid using these Generic models as these brackets are already included in the Communications Devices Families. Also note that you may create Schedules listing all models in your project along with the correct mounting brackets if they are optional brackets.

After inserting, you may orient the loudspeaker as desired per instance using the orientation parameters under “Model Properties”

The Pivoting Wall Bracket Family includes Azimuth, Pitch, and a Yes/No switch (Horizontal) to rotate the enclosure 90°. The Yoke Bracket Family includes Azimuth and Rotation parameters. Both Families include a visibility switch “Remove Logo”.

The Family also includes the industry standard REP (Required Electrical Power) calculation to calculate the minimum amplifier power needed to achieve the level needed at the listener. Enter the level needed in the “Desired Level @ Listener” parameter, The distance from the loudspeaker to the farthest listener of interest in the “D2” parameter, and desired “Peak Factor”. The Family will then report the minimum power required in the “REP” parameter.